| Pin No (E10) J Spec pinouts Autobox ECU only Unused pin | Symbol | Connection | Wire colour | Pin No (E9) | Symbol | Connection | Wire colour | Pin No (E9) | Symbol | Connection | Wire colour |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | IGSW | Ignition Switch |  |  | NCO - | O/D direct clutch speed sensor |  |  | VCC | TPS and MAP feed |  |
|  | SP1 | No 1 Vehicle Speed Sensor |  |  |  |  |  |  | VTA2 | Sub-throttle position sensor |  |
|  | KD | Kick-Down Switch |  |  |  |  |  |  | VTA1 | Throttle position sensor |  |
|  | STP | Stop Light switch |  |  |  |  |  |  | THW | Engine coolant temp sensor |  |
|  | EGW | CAT temp warning lamp |  |  | G2 - | Camshaft positions sensor No.2 |  |  | THA | Intake air temp sensor |  |
|  | W | Malfunction indicator lamp |  |  | G1 - | Camshaft position sensor No 1 |  |  |  |  |  |
|  | R | Park/neutral position switch |  |  | NE - | Crankshaft position sensor |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | OX1 | Main heated oxygen sensor |  |
|  | 2 | Park/neutral position switch |  |  | S2 | A/T No 2 solenoid |  |  | KNK2 | No 2 knock sensor |  |
|  | L | Park/neutral position switch |  |  | S1 | A/T No 1 solenoid |  |  | KNK1 | No 1 knock sensor |  |
|  | ABS | ABS & TRAC ECU |  |  |  |  |  |  | FAIL | Traction control ECU |  |
|  |  |  |  |  | SLT - | A/T solenoid SLT |  |  | IGT6 | Igniter |  |
|  | TRC - | TRAC ECU |  |  | SLN - | A/T No 4 solenoid |  |  | IGT5 | Igniter |  |
|  | TRC + | TRAC ECU |  |  | SLU - | A/T No 3 solenoid |  |  | IGT4 | Igniter |  |
|  | ELS | Defogger relay, Taillight relay |  |  | #60 | No 6 injector |  |  | IGT3 | Igniter |  |
|  | TACO | Tachometer |  |  | #50 | No 5 injector |  |  | IGT2 | Igniter |  |
|  |  |  |  |  | #40 | No 4 injector |  |  | IGT1 | Igniter |  |
|  | M | A/T pattern select switch |  |  | #30 | No 3 injector |  |  | IGF | Igniter Feedback |  |
|  | TE2 | Data link connector 2 |  |  | #20 | No 2 injector |  |  |  |  |  |
|  | TE1 | Data link connector 1 and 2 |  |  | #10 | No 1 injector |  |  | PMC | VSV for wastegate valve |  |
|  | DI | Fuel pump ECU |  |  | NCO + | O/D direct clutch speed sensor |  |  |  |  |  |
|  | FPC | Fuel pump ECU |  |  |  |  |  |  | PM1 | Turbo pressure sensor |  |
|  | ACMG | A/C Magnet clutch relay |  |  |  |  |  |  | IDL2 | Sub throttle position sensor |  |
|  | M-REL | EFI main relay |  |  | OIL | A/T oil temp sensor |  |  | IDL1 | Throttle position sensor |  |
|  | MI | Manual indicator light |  |  | G2 | Camshaft position sensor No 2 |  |  | E2 | Sensor ground |  |
|  | EFI - | TRAC ECU |  |  | G1 | Camshaft position sensor No 1 |  |  |  |  |  |
|  | EFI + | TRAC ECU |  |  | NE | Crankshaft position sensor |  |  | EFIF | Traction control ECU |  |
|  | OD2 | O/D Main switch |  |  |  |  |  |  |  |  |  |
|  | CCO | Cat temp sensor input |  |  | VF1 | Data link connector 1 |  |  | E1 | ECM Ground |  |
|  | +B | EFI Main relay |  |  | SLT + | A/T solenoid SLT |  |  | HT1 | Main heated oxygen sensor |  |
|  |  |  |  |  | ISC4 | IAC valve |  |  |  |  |  |
|  | BATT | Battery |  |  | ISC3 | IAC valve |  |  |  |  |  |
|  | A/C | A/C amplifier |  |  | ISC2 | IAC valve |  |  | EVAP | VSV for EVAP |  |
|  |  |  |  |  | ISC1 | IAC valve |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | NSW | Park/neutral position sensor |  |
|  |  |  |  |  |  |  |  |  | STA | Starter relay |  |
|  | NEO | TRAC ECU |  |  | VSV3 | VSV for exhaust bypass valve |  |  |  |  |  |
|  | VTO2 | Sub-throttle position sensor |  |  | VSV2 | VSV for exhaust gas control valve |  |  | EO2 | Power ground |  |
|  | VTO1 | Throttle position sensor |  |  | VSV1 | VSV for intake air control valve |  |  | EO1 | Power ground |  |